June 7, 2012

| Parameter | Methodology | Approved Standard Methods in Current MUR | Standard Methods 22nd Edition Editorial Updates |
| --- | --- | --- | --- |
| 1. Acidity, as CaCO3, mg/L | Electrometric endpoint or phenolphthalein endpoint | 2310 B-1997 | 2310 B-2011 |
| 2. Alkalinity, as CaCO3, mg/L | Electrometric or Colorimetric titration to pH 4.5, Manual | 2320 B-1997 | 2320 B-2011 |
|  | Automatic |  |  |
| 3. Aluminum—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 D-1999 or 3111 E-1999 | 3111 D-2011 or 3111 E-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | Direct Current Plasma (DCP) |  |  |
|  | Colorimetric (Eriochrome cyanine R) | 3500–Al B-2001 | 3500–Al B-2011 |
| 4. Ammonia (as N), mg/L | Manual distillation or gas diffusion (pH > 11) followed by any of the following: | 4500–NH3 B-1997 | 4500–NH3 B-2011 |
|  | Nesslerization |  |  |
|  | Titration | 4500–NH3 C-1997 | 4500–NH3 C-2011 |
|  | Electrode | 4500–NH3 D-1997 or E-1997 | 4500–NH3 D-2011 or E-2011 |
|  | Manual phenate, salicylate, or other substituted phenols in Berthelot reaction based methods | 4500–NH3 F-1997 | 4500–NH3 F-2011 |
|  | Automated phenate, salicylate, or other substituted phenols in Berthelot reaction based methods | 4500–NH3 G-1997 or 4500–NH3 H-1997 | 4500–NH3 G-2011 |
|  | Automated electrode |  |  |
|  | Ion Chromatography |  |  |
| 5. Antimony—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 6. Arsenic—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA gaseous hydride | 3114 B-2009 or 3114 C-2009 | 3114 B-2011 or 3114 C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | Colorimetric (SDDC) | 3500–As B-1997 | 3500–As B-2011 |
| 7. Barium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 D-1999 | 3111 D-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 8. Beryllium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 D-1999 or 3111 E-1999 | 3111 D-2011 or 3111 E-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Colorimetric (aluminon) |  |  |
| 9. Biochemical oxygen demand (BOD5), mg/L | Dissolved Oxygen Depletion | 5210 B-2001 | 5210 B-2011 |
| 10. Boron—Total, mg/L | Colorimetric (curcumin) | 4500–B B -2000 | 4500–B B -2011 |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 11. Bromide, mg/L | Electrode |  |  |
|  | Ion Chromatography | 4110 B-2000, C-2000, D-2000 | 4110 B-2011, C-2011, D-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 12. Cadmium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B -1999 or C-1999 | 3111 B -2011 or C-2011 |
|  | AA furnace | 3113 B -2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Voltametry |  |  |
|  | Colorimetric (Dithizone) | 3500-Cd-D-1990 |  |
| 13. Calcium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Titrimetric (EDTA) | 3500–Ca-B-1997 | 3500–Ca B-2011 |
|  | Ion Chromatography |  |  |
| 14. Carbonaceous biochemical oxygen demand (CBOD5), mg/L | Dissolved Oxygen Depletion with nitrification inhibitor | 5210 B-2001 | 5210 B-2011 |
| 15. Chemical oxygen demand (COD), mg/L | Titrimetric | 5220 B -1997 or C-1997 | 5220 B -2011 or C-2011 |
|  | Spectrophotometric, manual or automatic | 5220 D-1997 | 5220 D-2011 |
| 16. Chloride, mg/L | Titrimetric: (silver nitrate) | 4500–Cl—B-1997 | 4500–Cl—B-2011 |
|  | (Mercuric nitrate) | 4500–Cl—C-1997 | 4500–Cl—C-2011 |
|  | Colorimetric: manual |  |  |
|  | Automated (Ferricyanide) | 4500–Cl—E-1997 | 4500–Cl—E-2011 |
|  | Potentiometric Titration | 4500–Cl—D-1997 | 4500–Cl—D-2011 |
|  | Ion Selective Electrode |  |  |
|  | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 17. Chlorine—Total residual, mg/L | Amperometric direct | 4500–Cl D-2000 | 4500–Cl D-2011 |
|  | Amperometric direct (low level) | 4500–Cl E-2000 | 4500–Cl E-2011 |
|  | Iodometric direct | 4500–Cl B-2000 | 4500–Cl B-2011 |
|  | Back titration ether end–point | 4500–Cl C-2000 | 4500–Cl C-2011 |
|  | DPD–FAS | 4500–Cl F-2000 | 4500–Cl F-2011 |
|  | Spectrophotometric, DPD | 4500–Cl G-2000 | 4500–Cl G-2011 |
|  | Electrode |  |  |
| 17A. Chlorine—Free Available, mg/L | Amperometric direct | 4500–Cl D-2000 | 4500–Cl D-2011 |
|  | Amperometric direct (low level) | 4500–Cl E-2000 | 4500–Cl E-2011 |
|  | DPD–FAS | 4500–Cl F-2000 | 4500–Cl F-2011 |
|  | Spectrophotometric, DPD | 4500–Cl G-2000 | 4500–Cl G-2011 |
| 18. Chromium VI dissolved, mg/L | 0.45–micron Filtration followed by any of the following: |  |  |
|  | AA chelation–extraction | 3111 C-1999 | 3111 C-2011 |
|  | Ion Chromatography | 3500–Cr C-2009 | 3500–Cr C-2011 |
|  | Colorimetric (Diphenyl–carbazide) | 3500–Cr B-2009 | 3500–Cr B-2011 |
| 19. Chromium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA chelation–extraction | 3111 C-1999 | 3111 C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP, |  |  |
|  | Colorimetric (Diphenyl–carbazide) | 3500–Cr B-2009 | 3500–Cr B-2011 |
| 20. Cobalt—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 21. Color, platinum cobalt units or dominant wavelength, hue, luminance purity | Colorimetric (ADMI) |  |  |
|  | (Platinum cobalt) | 2120 B-2001 | 2120 B-2011 |
|  | Spectrophotometric |  |  |
| 22. Copper—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Colorimetric (Neocuproine) | 3500–Cu B-1999 | 3500–Cu B-2011 |
|  | Colorimetric (Bathocuproine) | 3500–Cu C-1999 | 3500–Cu C-2011 |
| 23. Cyanide—Total, mg/L | Automated UV digestion /distillation and Colorimetry |  |  |
|  | Segmented Flow Injection, In-Line Ultraviolet Digestion followed by gas diffusion amperometry |  |  |
|  | Manual distillation with MgCl2 followed by any of the following: | 4500–CN– B-1999 or C-1999 | 4500–CN– B-2011 or C-2011 |
|  | Flow Injection , gas diffusion amperometry |  |  |
|  | Titrimetric | 4500–CN– D-1999 | 4500–CN– D-2011 |
|  | Spectrophotometric, manual | 4500–CN– E-1999 | 4500–CN– E-2011 |
|  | Semi - Automated |  |  |
|  | Ion Chromatography |  |  |
|  | Ion Selective Electrode | 4500–CN– F-1999 | 4500–CN– F-2011 |
| 24. Cyanide-Available, mg/L | Cyanide Amenable to Chlorination (CATC); Manual distillation with MgCl2 followed by Titrimetric or Spectrophotometric | 4500–CN– G-1999 | 4500–CN– G-2011 |
|  | Flow injection and ligand exchange, followed by gas diffusion amperometry |  |  |
|  | Automated Distillation and Colorimetry (no UV digestion) |  |  |
| 24.A Cyanide-Free, mg/L | Flow Injection , followed by gas diffusion amperometry |  |  |
|  | Manual micro-diffusion and colorimetry |  |  |
| 25. Fluoride—Total, mg/L | Manual distillation followed by any of the following: | 4500–F– B-1997 | 4500–F– B-2011 |
|  | Electrode, manual | 4500–F– C-1997 | 4500–F– C-2011 |
|  | Electrode, automated |  |  |
|  | Colorimetric, (SPADNS) | 4500–F– D-1997 | 4500–F– D-2011 |
|  | Automated complexone | 4500–F– E-1997 | 4500–F– E-2011 |
|  | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 26. Gold—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration, | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace, | 3113 B-2004 |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 27. Hardness—Total, as CaCO3, mg/L | Automated colorimetric, |  |  |
|  | Titrimetric (EDTA) | 2340 C-1997 | 2340 C-2011 |
|  | Ca plus Mg as their carbonates, by inductively coupled plasma or AA direct aspiration. | 2340 B-1997 | 2340 B-2011 |
| 28. Hydrogen ion (pH), pH units | Electrometric measurement | 4500–H+-2000 | 4500–H+-2011 |
|  | Automated electrode |  |  |
| 29. Iridium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 30. Iron—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace | 3113 B -2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Colorimetric (Phenanthroline) | 3500–Fe B-1997 | 3500–Fe B-2011 |
| 31. Kjeldahl Nitrogen —Total, (as N), mg/L | Manual digestion and distillation or gas diffusion followed by any of the following:: | 4500–Norg B-1997 or C–1997 and 4500–NH3 B-1997 | 4500–Norg B-2011 or C–2011 and 4500–NH3 B-2011 |
|  | Titration | 4500–NH3 C-1997 | 4500–NH3 C-2011 |
|  | Nesslerization |  |  |
|  | Electrode | 4500–NH3 D-1997 or E-1997 | 4500–NH3 D-2011 or E-2011 |
|  | Semi-automated phenate | 4500–NH3 G-1997 | 4500–NH3 G-2011 |
|  |  | 4500 NH3 H-1994 | 4500 NH3 H-2011 |
|  | Manual phenate, salicylate, or other substituted phenols in Berthelot reaction based methods | 4500–NH3 F-1997 | 4500–NH3 F-2011 |
|  | Automated phenate, salicylate, or other substituted phenols in Berthelot reaction based methods colorimetric (auto digestion and distillation) |  |  |
|  | Semi-automated block digestor colorimetric (distillation not required) | 4500–Norg D-1997 | 4500–Norg D-2011 |
|  | Block digester, followed by Auto distillation and Titration |  |  |
|  | Block digester, followed by Auto distillation and Nesslerization, |  |  |
|  | Block Digester, followed by Flow injection gas diffusion (distillation not required) |  |  |
| 32. Lead—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Voltametry |  |  |
|  | Colorimetric (Dithizone) | 3500–Pb B-1997 | 3500–Pb B-2011 |
| 33. Magnesium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Gravimetric |  |  |
|  | Ion Chromatography |  |  |
| 34. Manganese—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Colorimetric (Persulfate) | 3500–Mn B-1999 | 3500–Mn B-2011 |
|  | Colorimetric (Periodate) |  |  |
| 35. Mercury—Total , mg/L | Cold vapor, Manual | 3112 -2009 | 3112 -2011 |
|  | Cold vapor, Automated |  |  |
|  | Cold vapor atomic fluorescence spectrometry (CVAFS) |  |  |
|  | Purge and Trap CVAFS |  |  |
| 36. Molybdenum—Total , mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 D-1999 | 3111 D-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 37. Nickel—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 38. Nitrate (as N), mg/L | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
|  | Ion Selective Electrode | 4500–NO3–D-2000 | 4500–NO3–D-2011 |
|  | Colorimetric (Brucine sulfate) |  |  |
|  | Nitrate-nitrite N minus Nitrite N |  |  |
| 39. Nitrate-nitrite (as N), mg/L | Cadmium reduction, Manual | 4500–NO3–E-2000 | 4500–NO3–E-2011 |
|  | Cadmium reduction, Automated | 4500–NO3–F-2000 | 4500–NO3–F-2011 |
|  | Automated hydrazine | 4500–NO3–H-2000 | 4500–NO3–H-2011 |
|  | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 40. Nitrite (as N), mg/L | Spectrophotometric: Manual | 4500–NO2–B-2000 | 4500–NO2–B-2011 |
|  | Automated (Diazotization) |  |  |
|  | Automated (*bypass cadmium reduction) | 4500–NO3–F-2000 | 4500–NO3–F-2011 |
|  | Manual (*bypass cadmium reduction) | 4500–NO3–E-2000 | 4500–NO3–E-2011 |
|  | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 41. Oil and grease—Total recoverable, mg/L | Hexane extractable material (HEM): n–Hexane extraction and gravimetry | 5520 B-2001 | 5520 B-2011 |
|  | Silica gel treated HEM (SGT–HEM): Silica gel treatment and gravimetry. | 5520 B-2001 and 5520 F-2001 | 5520 B-2011 and 5520 F-2011 |
| 42. Organic carbon—Total (TOC), mg/L | Combustion | 5310 B-2000 | 5310 B-2011 |
|  | Heated persulfate or UV persulfate oxidation | 5310 C 2000 | 5310 C 2011 |
|  |  | 5310 D 2000 | 5310 D 2011 |
| 43. Organic nitrogen (as N), mg/L | Total Kjeldahl N minus ammonia N |  |  |
| 44. Orthophosphate (as P), mg/L | Ascorbic acid method: |  |  |
|  | Automated | 4500-P F-1999 or G-1999 | 4500-P F-2011 or G-2011 |
|  | Manual single reagent | 4500-P E-1999 | 4500-P E-2011 |
|  | Manual two reagent |  |  |
|  | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 45. Osmium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration, | 3111 D-1999 | 3111 D-2011 |
|  | AA furnace |  |  |
| 46. Oxygen, dissolved, mg/L | Winkler (Azide modification) | 4500–O B-2001, C-2001, D-2001, E-2001, F-2001 | 4500–O B-2011, C-2011, D-2011, E-2011, F-2011 |
|  | Electrode | 4500–O G-2001 | 4500–O G-2011 |
| 47. Palladium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 48. Phenols, mg/L | Manual distillation (1) Followed by any of the following: | 5530 B-2005 |  |
|  | Colorimetric (4AAP) manual, | 5530 D-2005 (2) |  |
|  | Automated colorimetric (4AAP) |  |  |
| 49. Phosphorus (elemental), mg/L | Gas–liquid chromatography |  |  |
| 50. Phosphorus—Total, mg/L | Persulfate digestion followed by any of the following: | 4500-P B(5)-1999 | 4500-P B(5)-2011 |
|  | Manual | 4500-P E-1999 | 4500-P E-2011 |
|  | Automated ascorbic acid reduction | 4500-P F-1999, G-1999, H-1999 | 4500-P F-2011, G-2011, H-2011 |
|  | ICP/AES , | 3120 B-1999 | 3120 B-2011 |
|  | Semi–automated block digestor (TKP digestion) |  |  |
| 51. Platinum—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 52. Potassium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | Flame photometric | 3500–K B-1997 | 3500–K B-2011 |
|  | Electrode | 3500-K C-1997 | 3500-K C-2011 |
|  | Ion Chromatography |  |  |
| 53. Residue—Total, mg/L | Gravimetric, 103–105° | 2540 B-1997 | 2540 B-2011 |
| 54. Residue—filterable, mg/L | Gravimetric, 180° | 2540 C-1997 | 2540 C-2011 |
| 55. Residue—non–filterable (TSS), mg/L | Gravimetric, 103–105 °C post washing of residue | 2540 D-1997 | 2540 D-2011 |
| 56. Residue—settleable, mg/L | Volumetric, (Imhoff cone), or gravimetric | 2540 F-1997 | 2540 F-2011 |
| 57. Residue—Volatile, mg/L | Gravimetric, 550 °C | 2540-E-1997 | 2540-E-2011 |
| 58. Rhodium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration, or | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 59. Ruthenium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration, or | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 60. Selenium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | AA gaseous hydride | 3114 B-2009 or C-2009 | 3114 B-2011 or C-2011 |
| 61. Silica—Dissolved, mg/L | 0.45 micron filtration followed by any of the following: |  |  |
|  | Colorimetric, Manual | 4500–SiO2 C-1997 | 4500–SiO2 C-2011 |
|  | Automated (Molybdosilicate) | 4500–SiO2 E-1997 or F-1997 | 4500–SiO2 E-2011 or F-2011 |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 62. Silver—Total, , mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
| 63. Sodium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP |  |  |
|  | Flame photometric | 3500–Na B-1997 | 3500–Na B-2011 |
|  | Ion Chromatography |  |  |
| 64. Specific conductance, micromhos/cm at 25°C | Wheatstone bridge | 2510 B-1997 | 2510 B-2011 |
| 65. Sulfate (as SO4), mg/L | Automated colorimetric | 4500-SO42- F-1997 or G-1997 | 4500-SO42- F-2011 or G-2011 |
|  | Gravimetric | 4500-SO42- C-1997 or D-1997 | 4500-SO42- C-2011 or D-2011 |
|  | Turbidimetric | 4500-SO42- E-1997 | 4500-SO42- E-2011 |
|  | Ion Chromatography | 4110 B-2000 or C-2000 | 4110 B-2011 or C-2011 |
|  | CIE/UV | 4140 B-1997 | 4140 B-2011 |
| 66. Sulfide (as S), mg/L | Sample Pretreatment | 4500–S2– B, C -2000 | 4500–S2– B, C -2011 |
|  | Titrimetric (iodine) | 4500–S2–F-2000 | 4500–S2–F-2011 |
|  | Colorimetric (methylene blue) | 4500–S2–D-2000 | 4500–S2–D-2011 |
|  | Ion Selective Electrode | 4500–S2–G-2000 | 4500–S2–G-2011 |
| 67. Sulfite (as SO3), mg/L | Titrimetric (iodine-iodate) | 4500–SO32–B-2000 | 4500–SO32–B-2011 |
| 68. Surfactants, mg/L | Colorimetric (methylene blue) | 5540 C-2000 | 5540 C-2011 |
| 69. Temperature, °C | Thermometric | 2550 B-2000 |  |
| 70. Thallium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 71. Tin—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 | 3111 B-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | STGFAA |  |  |
|  | ICP/AES |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 72. Titanium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 D-1999, | 3111 D-2011 |
|  | AA furnace |  |  |
|  | DCP |  |  |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
| 73. Turbidity, NTU | Nephelometric | 2130 B-2001 | 2130 B-2011 |
| 74. Vanadium—Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 D-1999 | 3111 D-2011 |
|  | AA furnace | 3113 B-2004 |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP, |  |  |
|  | Colorimetric (Gallic Acid) | 3500–V B-1997 | 3500–V B-2011 |
| 75. Zinc –Total, mg/L | Digestion followed by any of the following: |  |  |
|  | AA direct aspiration | 3111 B-1999 or C-1999 | 3111 B-2011 or C-2011 |
|  | AA furnace |  |  |
|  | ICP/AES | 3120 B-1999 | 3120 B-2011 |
|  | ICP/MS | 3125 B-2009 | 3125 B-2011 |
|  | DCP, |  |  |
|  | Colorimetric (Dithizone) |  |  |
|  | (Zincon) | 3500–Zn -1997 | 3500–Zn -2011 |

| Table IC.–List of Approved Pesticide Test Procedures and Editorial Change |  |  |  |
| --- | --- | --- | --- |
| Parameter | Method | Approved Standard Methods | Standard Methods 22nd Edition Editorial Updates |
| 1. Acenaphthene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 2. Acenaphthylene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 3. Acrolein | GC |  |  |
|  | GC/MS |  |  |
| 4. Acrylonitrile | GC |  |  |
|  | GC/MS |  |  |
| 5. Anthracene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 6. Benzene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 7. Benzidine |  |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC |  |  |
| 8. Benzo(a)anthracene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440B-00 |  |
| 9. Benzo(a)pyrene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 10. Benzo(b)fluor-anthene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 11. Benzo(g,h,i) perylene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 12. Benzo(k) fluoranthene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 13. Benzyl chloride | GC |  |  |
|  | GC/MS |  |  |
| 14. Benzyl butyl phthalate | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 15. Bis(2-chloroethoxy) methane | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 16. Bis(2-chloroethyl) ether | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 17. Bis(2-ethylhexyl) phthalate | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 18. Bromodichloromethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 19. Bromoform | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 20. Bromomethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 21. 4-Bromophenyl phenyl ether | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 22. Carbon tetrachloride | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 23. 4-Chloro-3-methyl phenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 24. Chlorobenzene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 25. Chloroethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 26. 2-Chloroethylvinyl ether | GC |  |  |
|  | GC/MS |  |  |
| 27. Chloroform | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 28. Chloromethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 29. 2-Chloronaphthalene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 30. 2-Chlorophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 31. 4-Chlorophenyl phenyl ether | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 32. Chrysene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 33. Dibenzo(a,h)anthracene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 34. Dibromochloromethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 35. 1,2-Dichlorobenzene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 36. 1,3-Dichlorobenzene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 37. 1,4-Dichlorobenzene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 38. 3,3-Dichlorobenzidine | GC/MS | 6410 B-00 |  |
|  | HPLC |  |  |
| 39. Dichlorodifluoromethane | GC |  |  |
|  | GC/MS |  |  |
| 40. 1,1-Dichloroethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 41. 1,2-Dichloroethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 42. 1,1-Dichloroethene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 43. trans-1,2-Dichloroethene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 44. 2,4-Dichlorophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 45. 1,2-Dichloropropane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 46. cis-1,3-Dichloropropene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 47. trans-1,3-Dichloropropene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 48. Diethyl phthalate | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 49. 2,4-Dimethylphenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 50. Dimethyl phthalate | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 51. Di-n-butyl phthalate | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 52. Di-n-octyl phthalate | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 53. 2,3-Dinitrophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 54. 2,4-Dinitrotoluene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 55. 2,6-Dinitrotoluene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 56. Epichlorohydrin | GC |  |  |
|  | GC/MS |  |  |
| 57. Ethylbenzene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 58. Fluoranthene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 59. Fluorene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 60. 1,2,3,4,6,7,8- Heptachloro-dibenzofuran | GC/MS |  |  |
| 61. 1,2,3,4,7,8,9-Heptachloro-dibenzofuran | GC/MS |  |  |
| 62. 1,2,3,4,6,7,8- Heptachlorodibenzo-p-dioxin | GC/MS |  |  |
| 63. Hexachlorobenzene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 64. Hexachloro-butadiene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 65. Hexachlorocyclopentadiene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 66. 1,2,3,4,7,8- Hexachlorodibenzofuran | GC/MS |  |  |
| 67. 1,2,3,6,7,8- Hexachlorodibenzofuran | GC/MS |  |  |
| 68. 1,2,3,7,8,9- Hexachlorodibenzofuran | GC/MS |  |  |
| 69. 2,3,4,6,7,8- Hexachlorodibenzofuran | GC/MS |  |  |
| 70. 1,2,3,4,7,8- Hexachlorodibenzo-p-dioxin | GC/MS |  |  |
| 71. 1,2,3,6,7,8- Hexachlorodibenzo-p-dioxin | GC/MS |  |  |
| 72. 1,2,3,7,8,9- Hexachlorodibenzo-p-dioxin | GC/MS |  |  |
| 73. Hexachloroethane | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 74. Ideno(1,2,3-cd) pyrene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 75. Isophorone | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 76. Methylene chloride | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 77. 2-Methyl-4,6-dinitrophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 78. Naphthalene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 79. Nitrobenzene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC |  |  |
| 80. 2-Nitrophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 81. 4-Nitrophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 82. N-Nitroso-dimethylamine | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 83. N-Nitrosodi-n-propylamine | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 84. N-Nitroso-diphenylamine | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 85. Octachloro-dibenzofuran | GC/MS |  |  |
| 86. Octachlorodibenzo-p-dioxin | GC/MS |  |  |
| 87. 2,2'-Oxybis(2-chloropropane) [also known as bis(2-chloroisopropyl) ether] | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 88. PCB-1016 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 89. PCB-1221 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 90. PCB-1232 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 91. PCB-1242 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 92. PCB-1248 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 93. PCB-1254 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 94. PCB-1260 | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 95. 1,2,3,7,8- Pentachloro-dibenzofuran | GC/MS |  |  |
| 96. 2,3,4,7,8- Pentachloro-dibenzofuran | GC/MS |  |  |
| 97. 1,2,3,7,8,- Pentachlorodibenzo-p-dioxin | GC/MS |  |  |
| 98. Pentachlorophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 99. Phenanthrene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 100. Phenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 101. Pyrene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
|  | HPLC | 6440 B-00 |  |
| 102. 2,3,7,8- Tetra-chlorodibenzofuran | GC/MS |  |  |
| 103. 2,3,7,8- Tetra-chlorodibenzo-p-dioxin | GC/MS |  |  |
| 104. 1,1,2,2-Tetra-chloro ethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 105. Tetrachloroethene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 106. Toluene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 107. 1,2,4-Trichloro-benzene | GC |  |  |
|  | GC/MS | 6410 B-00 |  |
| 108. 1,1,1-Trichloro-ethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 109. 1,1,2-Trichloro-ethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 110. Trichloroethene | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 111. Trichlorofluoromethane | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |
| 112. 2,4,6-Trichlorophenol | GC | 6420 B-00 |  |
|  | GC/MS | 6410 B-00 |  |
| 113. Vinyl chloride | GC | 6200 C-97 | 6200 C-11 |
|  | GC/MS | 6200 B-97 | 6200 B-11 |